ISO/IEC JTC1/SC29/WG1 M726

19 December 1997

ISO/IEC JTC1/SC29/WG1

(ITU-T SG8)

Coding of Still Pictures

JBIG	JPEG

Joint Bi-level Image	Joint Photographic

Experts Group	Experts Group

TITLE: 	Error Pattern Generation Programs for Core Experiments on JPEG-2000 Error Resilience :

SOURCE: 	NTT Mobile Communications Network Inc. (NTT DoCoMo)

Author:	Toshio Miki and Toshiro Kawahara

PROJECT: JTC 1.29.14 (15444)

STATUS: Proposal

REQUESTED

ACTION: Technology contributions

DISTRIBUTION: WG1 Mailing List, SC29 committee, unlimited distribution

Contact:

Toshio MIKI

NTT Mobile Communications Network Inc.

1-1 Hikarinooka, Yokosuka-shi, Kanagawa, 239 Japan

Tel: +81 468 59 3490, Fax: +81 468 57 7909, E-mail: miki@mlab.nttdocomo.co.jp

# 1. Introduction

This document describes the error generation software for core experiments on JPEG-2000 error resilience.  A similar program was provided for core experiments on MPEG-4 Video/Audio error resilience by MPEG96/M1492[2], assuming simple models of error patterns supplied to the coding layer from the multiplexing layer and the physical transmission layer.  It would be a great benefit to use the error conditions identical with MPEG-4 Video/Audio as a reasonable shortcut.

In this document, two types of modeled error pattern generation programs are described, namely erand and gilbert.  erand creates a bit error sequence corresponding to random errors, while gilbert to burst errors.  These error patterns are very frequently observed in the actual mobile radio channels and base band modem transmission.  Their source programs are attached with the document.

Note that so-called packet loss program is not introduced here, since it requires the frame structure of coded bitstream and interface definition between coding layer and underlying layers.  That would be provided later when these specifications are given.

First of all, the background that the artificial simple error models was introduced in MPEG-4 Video is summarized in section 2, by referring WG11/N1223. [1]  Detailed software description and its usage for each error model is described in section 3.

# 2. Background

As described in [1], there were exhaustive discussions on what error patterns should be used in the core experiments in MPEG-4 Video error resilience.  The major points were as follows:

- realistic errors collected in the field, or simplified model-based errors

- what type of channels should be considered

In the absence of underlying layer’s definition, in particular, the multiplexing layer, and in order to avoid to tune a specific channel, the MPEG-4 error resilience video people concluded to employ simple models of error patterns as typical ones supplied to the source coding layer.  They consist of Random errors, Burst errors with two-state Markov model, and Random packet loss with a variety of length.  These models cover a wide range of actual channels, for instance, wireless transmission systems like GSM/PDC/USDC/DECT/PHS/(IMT-2000), internet, base-band modems, and so on.  Detailed specifications, such as bit error rate, burst error length, and packet loss length were decided later as written in [4].  The error generation programs were provided by the authors, according to the agreement.

The above approach is thought as the first phase.  The second phase should consider more realistic error conditions, taking actual error patterns obtained in transmission channels as well as the multiplexing layer into account.  This will be done in the verification test scheduled next March.

# 3. Software description

The software can generate three classes of error conditions, random error and burst error with 2 state Markov model (or Gilbert model). The name of the programs are erand and gilbert, respectively.

We briefly describe the usage of these two programs below. The terms <Input file> and <Output file> in the following description means the name of input sequence file and output sequence file respectively. You can use ‘-‘ for the filename, which means standard input/output. All the sequence files processed with these programs must be in binary form. The transmitted bits should be packed into octets so that their MSB (most significant bit) is transmitted first.

## 3.1. Random error (erand)

This software injects random bit error under specified bit error rate. The usage of this program is:

- Usage

% erand <iseed> <Bitrate(bps)> <BER> <Input file> <Output file>

- Example

Initial seed		: 10

Bitrate 			: 64k bits/sec

Bit error rate		: 1.0e-3

% erand 10 64000 1.0e-3 input.seq output.seq

The parameter <Bitrate> would not be used, unless users intentionally revise the program to introduce the initial error free period as done in Video SG.

## 3.2 Burst error (gilbert)

This software injects burst error generated by Gilbert model. The usage of this program is:

- Usage

% gilbert <iseed> <Bitrate(bps)> <Average BER> <Burst len(ms)> \

<Burst BER> <Input file> <Output file>

- Example

Initial seed		: 10

Bitrate 			: 64k bits/sec

Average bit error rate	: 1.0e-3

Average burst length	: 10 ms

BER in burst duration	: 50%

% gilbert 10 64000 1.0e-3 10 0.5 input.seq output.seq

For low bitrate and low average BER conditions, we should choose appropriate iseed carefully. For this purpose, this program reports the statistics of the generated average BER, average error free length and average burst length.  Some examples of iseed used in Video SG is given in MPEG97/M2444. [5]

BER in burst duration could be reduced to lower than 50%, but note that 50% has been used in Video SG as a typical value.

## 3.3 Copyright & Warranty

The copyright of these programs belongs to NTT Mobile Communications Network Inc.(NTT DoCoMo).  As long as they are used for the purpose of MPEG-4 and JPEG-2000 standardization activities, NTT DoCoMo is willing to provide them without any licensing procedure.  No warranty is served for these programs.

# Refferences

[1] “Report on Error Conditions for Error Resilience Core Experiments”, ISO/IEC JTC1/SC29/WG11, N1223.

[2] Toshio Miki, Toshiro Kawahara and Tomoyuki Ohya, “Revised Error Pattern Generation Programs for Core Experiments on Error Resilience”, ISO/IEC JTC1/SC29/WG11, MPEG96/M1492.

[3] Toshio Miki, Toshiro Kawahara and Sanae Hotani, “Error Pattern Generation Programs for Core Experiments on MPEG-4 Audio Error Resilience”, ISO/IEC JTC1/SC29/WG11, MPEG97/M2686.

[4] “Description of Error Resilient Core Experiments”, ISO/IEC JTC1/SC29/WG11, N1808, July, 1997.

[5] Toshio Miki, Tomoyuki Ohya and Satoru Adachi, “Results of the Core Experiments on Error Resilience”, ISO/IEC JTC1/SC29/WG11, MPEG97/M2444, July, 1997.